# Network Design and Configuration Documentation 1. Hierarchical Network Design Model (LAN)

This network was designed using the hierarchical model which consists of three layers: Core, Distribution, and Access layers. This model improves scalability, performance, and fault isolation.

## Layers Description

Core Layer: Provides high-speed backbone connectivity between distribution devices.

Distribution Layer: Performs routing, filtering, and policy enforcement.

Access Layer: Connects end devices such as PCs, printers, and IP phones.

# Subnetting and IP Address Allocation

Subnetting was implemented to divide the network into smaller logical segments for better traffic management, security, and IP address utilization.

## Steps Used

- Determined number of required subnets.

- Calculated host requirements per subnet.

- Selected appropriate subnet mask.

- Assigned IP ranges to departments.

- Reserved IP addresses for servers and network devices.

# VLAN Configuration and Inter-VLAN Routing

VLANs were configured to segment departments for security and broadcast control. Each department was assigned a unique VLAN ID.

## Switch Configuration Example

enable configure terminal vlan 10 name HR vlan 20 name FINANCE vlan 30 name IT exit

## Assign Ports to VLAN

interface fastEthernet 0/1 switchport mode access switchport access vlan 10

## Trunk Configuration (802.1Q)

interface gig0/1 switchport trunk encapsulation dot1q switchport mode trunk

## Inter-VLAN Routing (Router-on-a-Stick)

interface gig0/0.10 encapsulation dot1Q 10 ip address 192.168.10.1 255.255.255.0

# Server Configuration

## DHCP Server

ip dhcp pool HR network 192.168.10.0 255.255.255.0 default-router 192.168.10.1 dns-server 192.168.10.2

DNS Server

Configured to resolve domain names to IP addresses for internal services.

Web Server

Hosts internal or public web applications accessible through HTTP/HTTPS.

Email Server

Configured for sending and receiving organizational emails using SMTP/POP3/IMAP.

# Dynamic Routing Protocol Configuration (OSPF)

OSPF was implemented for WAN routing to allow dynamic exchange of routing information between routers.

router ospf 1

network 10.0.0.0 0.0.0.255 area 0

# User Privileges and Password Security

Access security was implemented by configuring encrypted passwords for console, user EXEC, and privileged EXEC modes.

## Configuration Example

enable secret StrongPassword line console 0 password ConsolePass login

service password-encryption

# Conclusion

This document summarizes the structured approach used to design and configure the network. The implementation ensures scalability, security, efficient routing, and centralized network services.