Design and Implementation of a SOHO Network

# Project Overview

This document describes the design and implementation of a Small Office Home Office (SOHO) network using Cisco devices. The network consists of one router and one switch supporting three departments: Admin, IT, and Customer Service/Reception. Each department operates within its own VLAN, has wireless connectivity, receives automatic IP addressing, and can communicate with other departments.

# Network Requirements

- One router and one switch (Cisco devices).

- Three departments: Admin, IT, Customer Service/Reception.

- Each department must be placed in a separate VLAN.

- Each department must have wireless access.

- All hosts must receive IPv4 automatically.

- Inter-department communication must be enabled.

- Base network provided by ISP: 192.168.1.0.

# Technologies Implemented

- Simple network creation using router and access switch.

- Correct cabling for device interconnection.

- VLAN creation and port assignment.

- Subnetting and IP addressing.

- Router-on-a-stick inter-VLAN routing.

- Router configured as DHCP server.

- Wireless network configuration using access point.

- Host configuration.

- Testing and verification.

# Network Design

## IP Addressing Plan

The base network 192.168.1.0 was subnetted into three subnets:

Admin VLAN 10: 192.168.1.0 /26

IT VLAN 20: 192.168.1.64 /26

Customer Service VLAN 30: 192.168.1.128 /26

## VLAN Configuration (Switch)

enable configure terminal vlan 10 name ADMIN vlan 20 name IT vlan 30 name CUSTOMER

## Assigning Ports

interface range fa0/1-5 switchport mode access switchport access vlan 10

interface range fa0/6-10 switchport mode access switchport access vlan 20

interface range fa0/11-15 switchport mode access switchport access vlan 30

## Trunk Link to Router

interface g0/1 switchport mode trunk

## Router-on-a-Stick Configuration

interface g0/0.10 encapsulation dot1Q 10 ip address 192.168.1.1 255.255.255.192

interface g0/0.20 encapsulation dot1Q 20 ip address 192.168.1.65 255.255.255.192

interface g0/0.30 encapsulation dot1Q 30 ip address 192.168.1.129 255.255.255.192

## DHCP Configuration

ip dhcp pool ADMIN network 192.168.1.0 255.255.255.192 default-router 192.168.1.1 ip dhcp pool IT

network 192.168.1.64 255.255.255.192 default-router 192.168.1.65

ip dhcp pool CS network 192.168.1.128 255.255.255.192 default-router 192.168.1.129

## Wireless Configuration

Each department access point is configured with unique SSID mapped to its VLAN. Security mode WPA2 is recommended.

SSID: ADMIN_WIFI -> VLAN 10

SSID: IT_WIFI -> VLAN 20

SSID: CS_WIFI -> VLAN 30

## Host Configuration

All hosts are configured to obtain IP addresses automatically using DHCP. No static configuration is required.

## Testing and Verification

ping 192.168.1.65 ping 192.168.1.129 show ip route show vlan brief show ip dhcp binding

# Conclusion

The SOHO network was successfully designed and implemented according to requirements. VLAN segmentation ensures security, DHCP automates addressing, and inter-VLAN routing enables communication across departments. Testing confirmed full connectivity and proper operation.